Week 6: 10/11/2021 – Wednesday

- Outline of Meeting

The meeting was held on Zoom. Following from last week’s topic, the meeting was focused on characterizing a KID. Building from last week’s theory, this week extends for characterizing the measurement of a KID. By using the scattering parameters equations for the KID, by determining the ABCD values which are related to the transmission line, the S21 value can be found for the detector. This week’s task was focused on using a Python function given by my supervisor and given parameters to model a KID for a certain temperature range. The result of which is a plot of cascading frequencies each curve corresponding to a detector.

- Specification of Task

- Find Lint using Mattis Bardeen Approximations for Aluminium (use values given last time) at a temperature of 0.2K. Multiply this by “Squares” to get total Lint.

- Find the IDC value for the lowest temperature and make this fixed. Found from F0 and (Lint+Lgeo) and C_couple.

ω0=1(Lint+LGeo)(CIDC+CCouple)

- Run simulations code below to obtain S21 and plot this as a function of frequency

- Define a temperature step (say 0.02K) and calculate a new Lint using Mattis Bardeen Approximations at this slightly higher temperature.

- Simulate again changing only the value of Lint (IDC, L_geo and C_couple remain fixed)

- Repeat for all temperatures up to 0.35K

- Outline of Theory and Task Methodology

Last week discussed about the scattering parameters of the KID. This week, we used the Mattis Bardeen Approximations to determine the population change of the electrons and apply this to Lint for the change in internal inductance for a range of temperatures. Following this, the KID can be modelled for a temperature change, corresponding to a detection.

Detailed steps:

- First, we used the Mattis-Bardeen Approximation for a range of temperatures (from 0.2 to 0.35K with step of 0.02K) and constants defined. Equations:

σ1σn=2∆(T)ℏωexp(-∆0kBTK0(ℏω2kBT)[2sinh⁡(ℏω2kBT)]

σ2σn=π∆(T)ℏω[1-2exp(-∆0kBTexp-ℏω2kBTI0(ℏω2kBT)]

- From this, we can calculate the change in σ2 and σ1 for a change in temperature.

- Following this, we can substitute the values of σ2 into the LPD expression:

σs=-jnse2ωm

We take the magnitude of this, so the imaginary number can be ignored. Using this, found values for Cooper-pair popuation ns.

- Then, the value of ns is substituted into the LPD expression to obtain the LPD:

λL= mμ0nse2

- Using the LPD, substitute into the expression for Lint:

Lint=μ0λ2cotht2λ⁡

- Next, we can find the resistive part R using the following expression:

R=Lkωσ1σ2

This R accounts for the loss faced by the current when propagating across the quasi-particle as this path is resistive compared to the superconducting ns.

- Finally, plug the R and Lint values into the Python function given to obtain S21 and plot the amplitude of S21 against frequency. This figure is generated in the following section.

- Plot of S21 vs Temperature

This figure illustrates the S21 Amplitude variation with temperature. This is effectively a model of the detector, and this allows us to study and understand the effects of the parameters on its output.

- Python Function Given:

def Capacitive_Res_Sim(F0, C_couple, Z0, L_geo, L_int, Res, Sweep_BW, Sweep_points, Capacitance):

j=complex(0,1)

Cc=C_couple

F_min=F0-(Sweep_BW/2.0)

F_max=F0+(Sweep_BW/2.0)

Sweep=np.linspace(F_min, F_max, Sweep_points)

W=Sweep*2.0*pi

W0=2.0*pi*F0

L=L_geo+L_int

C=Capacitance

Zres= 1.0/((1./((j*W*L)+Res))+(j*W*C))   # Impedance of resonator section

Zc=1.0/(j*W*Cc)  #impedance of coupler

ZT=Zres+Zc

YT=1.0/ZT

S21 = 2.0/(2.0+(YT*Z0))

I_raw=S21.real

Q_raw=S21.imag

shift=((1.0-min(I_raw))/2.0)+min(I_raw)

I_cent=I_raw-shift

Q_cent=Q_raw

Phase=Atan(abs(Q_cent/I_cent))

QU=(W0*L)/Res

QL=(C*2)/(W0*(Cc**2)*Z0)

S21_Volt=abs(S21)

I_offset=shift

return  (Sweep, S21_Volt, Phase, I_raw, Q_raw, I_cent, Q_cent, QU, QL, I_offset)

- Python Code for Task

#imports

import numpy as np

import matplotlib.pyplot as plt

import scipy.constants as const

from scipy.special import iv as I0

from scipy.special import kv as K0

#Define Global Variables

L_geo = 55.6e-9

Z0 = 50.0

F0_base = 0.95e9        #At lowest Temp

squares= 27223

c_couple = 1.5e-14

TC = 1.5

Delta_0 = (3.5*const.Boltzmann*TC)/2

sigma_n = 6.0e7          # Normal stae conductvity if superconducting film

Thick = 20e-9            # Thickness of superconducting fil

w = 2 * np.pi * F0_base

me = const.m_e

miu_0 = 4*np.pi*10**-7

pi = np.pi

#Main code

def main():

#Define temperature range with step 0.01K

step = 0.02

temp = np.arange(0.20, 0.35, step)

#Find sigma1 and sigma 2 and Lint

sigma1, sigma2 = find_sigma1_sigma2(sigma_n ,Thick, TC, Delta_0, w, temp)

Lint = find_Lint_square(Thick, w, sigma2) * squares

#Find lk

Lk = find_lk(Thick, w, sigma2)

#Find Res

sigma12Ratio = sigma1/sigma2

Res = Lk*w*sigma12Ratio *squares

#IDC for Lowest Temp (0.2K)

Ltot_lowest = Lint[0] + L_geo

IDC = find_IDC(w, Ltot_lowest, c_couple)

#Find S21

Sweep_points = 20000

BW = 5e6

I_raw = np.zeros((Sweep_points, len(temp)), dtype="float")

Q_raw = np.copy(I_raw)

Phase = np.copy(Q_raw)

S21_Volt = np.copy(I_raw)

for i in range(0, len(Lint)):

Sweep, S21_Volt[:,i], Phase[:,i], I_raw[:,i], Q_raw[:,i],_,_,_,_,_ = Capacitive_Res_Sim(F0_base, c_couple, Z0, L_geo, Lint[i], Res[i], BW, Sweep_points, IDC)

plt.plot(Sweep/1e9, S21_Volt[:,i], label=str("{:.2f}".format(temp[i])))

#Graph labels and title

plt.legend(loc='center left', bbox_to_anchor=(1, 0.5), fancybox=True, title="Temperature / K")

plt.xlabel('Frequency / GHz', fontsize=13)

plt.ylabel('S21 Amplitude / V', fontsize=13);

plt.title("S21 Amplitude For Varying Temperatures")

plt.xlim(0.9490, 0.9505)

plt.locator_params(nbins=6)

plt.savefig("S21 Plot with Resistance")

plt.rcParams['figure.dpi'] = 300

#KID Simulating Function

def Capacitive_Res_Sim(F0, C_couple, Z0, L_geo, L_int, Res, Sweep_BW, Sweep_points, Capacitance):

""" Help file here"""

j=complex(0,1)

Cc=C_couple

F_min=F0-(Sweep_BW/2.0)

F_max=F0+(Sweep_BW/2.0)

Sweep=np.linspace(F_min, F_max, Sweep_points)

W=Sweep*2.0*pi

W0=2.0*pi*F0

L=L_geo+L_int

C=Capacitance

Zres= 1.0/((1./((j*W*L)+Res))+(j*W*C))   # Impedance of resonator section

Zc=1.0/(j*W*Cc)  #impedance of coupler

ZT=Zres+Zc

YT=1.0/ZT

S21 = 2.0/(2.0+(YT*Z0))

I_raw=S21.real

Q_raw=S21.imag

shift=((1.0-min(I_raw))/2.0)+min(I_raw)

I_cent=I_raw-shift

Q_cent=Q_raw

Phase=Atan(abs(Q_cent/I_cent))

QU=(W0*L)/Res

QL=(C*2)/(W0*(Cc**2)*Z0)

S21_Volt=abs(S21)

I_offset=shift

return  (Sweep, S21_Volt, Phase, I_raw, Q_raw, I_cent, Q_cent, QU, QL, I_offset)

#Function to find sigma1 and sigma2

def find_sigma1_sigma2(sigma_n ,Thick, TC, Delta_0, w, T):

#An interpolation formula for delta_T

delta_T = Delta_0*np.tanh(1.74*np.sqrt((TC/T)-1))

#Define constants to simplify eqn

multiplying_constant = delta_T/(const.hbar * w)

e_const_1 = - Delta_0/(const.Boltzmann*T)

e_const_2 = (const.hbar*w)/(2*const.Boltzmann*T)

#Parts of the sigma1 Ratio

A = 2*multiplying_constant

B = np.exp(e_const_1)

C = K0(0, e_const_2)

D = 2*(np.sinh(e_const_2))

#Find Sigma 1 and Sigma 2

sigma1Ratio = A * B * C * D

sigma2Ratio = np.pi*multiplying_constant*(1 - (2*np.exp(e_const_1)*np.exp(-e_const_2)*I0(0,e_const_2)))

sigma2 = sigma2Ratio * sigma_n

sigma1 = sigma1Ratio * sigma_n

return sigma1, sigma2

def find_lk(Thick, w, sigma2):

#Depth

lower_fraction = miu_0*sigma2*w

Lambda_T_MB = (1/lower_fraction)**0.5

fraction = Thick/(2*Lambda_T_MB)

#Terms for lk

A = (miu_0*Lambda_T_MB)/4

B = coth(fraction)

C = fraction*(csch(fraction))**2

#R vs T

lk = A*(B+C)

return lk

def find_Lint_square(Thick, w, sigma2):

#Depth

lower_fraction = miu_0*sigma2*w

Lambda_T_MB = (1/lower_fraction)**0.5

#Internal Inductance

fraction = Thick/(2*Lambda_T_MB)

L_int = (miu_0*Lambda_T_MB/2)*coth(fraction)

return L_int

#Define coth and csch

def coth(x):

return np.cosh(x)/np.sinh(x)

def csch(x):

return 1/np.sinh(x)

def Atan(x):

return np.arctan(x)

#Find IDC function

def find_IDC(w0, Ltot, Cc):

IDC = 1/((w0**2)*Ltot) - Cc

return IDC

def Magic_Formula(di, dq, didf, dqdf):

return (di*didf + dq*dqdf)/(didf**2 + dqdf**2)

main()